# Fuse Series Technical Specifications

## Selective Laser Sintering 3D Printers

## Quick Stats

Contact Sales

Printer

Fuse 1

Fuse 1+ 30W

Technology

Selective Laser Sintering

Selective Laser Sintering

Build Volume (W x D x H)

165 × 165 × 300 mm

6.5 x 6.5 x11.8 in

165 × 165 × 300 mm

6.5 x 6.5 x11.8 in

Layer Thickness

110 microns

0.004 in

110 microns

0.004 in

Laser Type

Ytterbium Fiber 10W

Ytterbium Fiber 30W

Laser Spot Size (FWHM)

200 microns

0.0079 in

247 microns

0.0097 in

Build Chamber

Modular, compatible with Fuse 1 generation printers and Fuse Sift

Modular, compatible with Fuse 1 generation printers and Fuse Sift

## Printing Properties

Technology

Selective Laser Sintering

Selective Laser Sintering

Build Volume (W × D × H)

16.5 x 16.5 x 30 cm

6.5 x 6.5 x 11.8 in

16.5 x 16.5 x 30 cm

6.5 x 6.5 x 11.8 in

Layer Thickness (Axis Resolution)

110 microns

0.004 in

110 microns

0.004 in

Hopper Capacity

17.8 Liters

14.5 Liters

Dimensions

68.5 x 64.5 x 106.5 cm

27 x 25.4 x 41.9 in

68.5 x 64.5 x 106.5 cm

27 x 25.4 x 41.9 in

Supports

No supports

No supports

Design/CAD Specifications

Download Fuse Series Design Guide (PDF)

Download Fuse Series Design Guide (PDF)

## Hardware

Minimum Access Dimensions (W × D × H)

125.5 × 149.5 × 187 cm

49.4 × 59.0 × 73.6 in

125.5 × 149.5 × 187 cm

49.4 × 59.0 × 73.6 in

Printer Dimensions (W × D × H)

64.5 × 68.5 × 107 cm (165.5 cm with stand)

25.4 × 27 × 42 in (65.0 in with stand)

64.5 × 68.5 × 107 cm (165.5 cm with stand)

25.4 × 27 × 42 in (65.0 in with stand)

Recommended Operating Footprint (W × D × H)

145.5 × 149.5 × 167.5 cm (49.4 × 59 × 66 in) to allow access to the front and sides of the printer.

145.5 × 149.5 × 167.5 cm (49.4 × 59 × 66 in) to allow access to the front and sides of the printer.

Stand Dimensions (W × D × H)

59.6 x 60 x 58.4 cm

23.5 × 23.6 × 23 in

59.6 x 60 x 58.4 cm

23.5 × 23.6 × 23 in

Weight

114 kg (without build chamber or powder)

251.3 lb (without build chamber or powder)

120 kg (without build chamber or powder)

265 pounds (without build chamber or powder)

Startup Time

< 60 minutes

< 60 minutes

Operating Environment

18 – 28 ºC

68 – 82 ºF

18 – 28 ºC

68 – 82 ºF

Internal Temperature

Up to 200 ºC

Up to 392 ºF

Up to 200 ºC

Up to 392 ºF

Temperature Control

Quartz tube heating elements

Positive temperature coefficient (PTC) cartridges

Quartz Tube Heating Elements

Resistive Air Heater

Air Handling

Pressure-controlled two-stage filtration

(Replaceable HEPA and carbon mediums)

Interface to external inert gas supply

Pressure controlled two-stage filtration

(Replaceable HEPA and carbon mediums)

Power Requirements

EU: 230 VAC, 7.5 A (dedicated circuit)

US: 120 VAC, 15 A (dedicated circuit)

EU: 230 VAC, 7.5 A (dedicated circuit)

US: 120 VAC, 15 A (dedicated circuit)

Galvanometers

Formlabs Custom

Formlabs Custom 2nd Generation

Laser Specifications

Ytterbium Fiber

IEC 60825-1: 2014 certified

1070 nm wavelength

Maximum 10 Watts

4.01 mrad beam divergence (nominal, full angle)

Ytterbium Fiber

IEC 60825-1: 2014 certified

1070 nm wavelength

Maximum 30 Watts

3.24 mrad beam divergence (nominal, full angle)

Laser Spot Size (FWHM)

200 microns

.0079 in

247 microns

.0097 in

Radiation Information

The Fuse 1 is a Class 1 Laser product. Accessible radiation is within Class 1 limits.

The Fuse 1+ 30W is a Class 1 Laser product. Accessible radiation is within Class 1 limits.

Connectivity

Wi-Fi (2.4 GHz)

Ethernet (1000 Mbit)

USB 2.0

Wi-Fi (2.4 GHz)

Ethernet (1000 Mbit)

USB 2.0

Printer Control

10.1” interactive touchscreen

1280 × 800 resolution

10.1” interactive touchscreen

1280 × 800 resolution

Alerts

Touchscreen alerts and tracking

SMS/email via Dashboard

Live video feed with computer vision

Proactive maintenance alerts

Touchscreen alerts and tracking

SMS/email via Dashboard

Live video feed with computer vision

Proactive maintenance alerts

## Software

Contact Sales

Print Preparation

PreForm Desktop Software

PreForm Desktop Software

System Requirements

Windows 7 (64-bit) and up

Mac OS X 10.12 and up

OpenGL 2.1

4 GB RAM (8 GB recommended)

Windows 7 (64-bit) and up

Mac OS X 10.12 and up

OpenGL 2.1

4 GB RAM (8 GB recommended)

Hardware Requirements

Fuse 1 generation 3D printer

Fuse 1 generation 3D printer

File Types

.STL or .OBJ

FORM file output

.STL or .OBJ

FORM file output